# AXI Performance Monitor IP — Design Request Specification

Purpose: This document defines the functional behavior, operating rules, and constraints for implementing an AXI performance monitor with bandwidth control and address protection in Verilog/SystemVerilog, for first delivery to the SoC IP team.
Audience: SoC and IP design, verification, and software engineers (assumes first receipt of this IP).

Scope: This document is the single normative baseline for functional and behavioral requirements of this IP. Register bitfields and addresses, pin/port lists, timing/electrical constraints, and SoC integration procedures are documented in companion material supplied by the IP provider, which MUST remain consistent with this specification.

Standalone delivery: This package (Korean and English markdown plus the block-diagram image) is sufficient to convey the requirements; no other repository markdown or internal documents need to be read.

### Recommended delivery bundle

| File | Description |
| --- | --- |
| request_for_design_busmon.md | Korean specification (normative prose and tables). |
| request_for_design_busmon_en.md | English specification of the same content. |
| busmon_architecture_overview.png | §10 overview figure; keep in the same folder as the markdown (relative image link). |

You MAY generate Word/PDF from markdown. The Mermaid diagram in §10.2 may not render in all viewers; §§4–9 and tables are normative.

## 0. Notation: Mandatory vs Designer Discretion

| Term | Meaning |
| --- | --- |
| MUST | Implement and verify as stated in this document. |
| SHOULD | Preferred practice; may be replaced if quality is equivalent or better. |
| MAY | Microarchitecture, implementation style, and detailed parameters are at designer discretion, provided required behavior and AXI protocol compliance are preserved. |

## 1. Design Goals (MUST)

- Deliver an AXI performance monitor IP suitable for production-quality use.

- Bandwidth/latency logging, bandwidth throttling (multiple threshold modes), external throttle control, and address protection MUST be controllable and observable via registers within one IP.

- Provide pipeline and timing margin appropriate for high-frequency operation.

## 2. Interfaces (MUST)

### 2.1 AXI

- AXI Master Up / Master Down (or Slave Up / Master Down naming MAY vary).

  - Upstream and downstream use the same data width and same clock.

- Default data width is 256 bit (other defaults MAY use parameters; 256 is the default).

- Optional register slices on Up and/or Down MUST be selectable via parameters.

### 2.2 Control / Status — APB

- Control/status registers are accessed through an APB 32-bit slave interface.

- pstrb MAY be assumed fixed to 0xF.

- Same clock domain as the AXI master interface.

### 2.3 Interrupt

- One interrupt output (shared across logging events or masked into one line — sources and mask bits are defined in §8 and in the companion register definition for this IP).

### 2.4 Off-bus Input — External Throttle Control

MUST

- Provide a 2-bit input port request_throttle[1:0] separate from AXI bus ports.

- Define legal encodings as 0 through 3 (treatment of unused encodings is MAY — document as ignore or same as 0).

- SHOULD operate in the same clock domain as AXI. If asynchronous, MAY add synchronizers; MUST consider metastability.

MAY

- Port naming and module-level pin prefixes.

## 3. Per-Feature Enable / Disable (MUST)

Control registers MUST provide per-feature enable/disable fields. Example split (field names and bit positions MAY vary):

- Bandwidth logging

- Latency logging (single-tx / multi-tx latency via sub-enables or combined statistics in period — §5.2)

- Bandwidth throttling

- External throttle control (request_throttle path — §6.3)

- Address protection

SHOULD reduce unnecessary clock toggles when features are off.

## 4. Transaction Filters (MUST)

Used consistently for logging, latency measurement, and throttle scope. Filters are combined with logical AND: a transaction is included only if all of address, ID, and direction conditions match (when the relevant feature is enabled).

### 4.1 Address Filter

MUST

- Through control registers, allow monitoring only transactions in a start address + size (or start/end) range, or full address space tracking.

MAY

- Address compare implementation (alignment rules, upper-bit masks, etc.).

### 4.2 ID Filter

MUST

- Support these modes for AXI ID (ARID/AWID width per AXI parameters), selectable by register:

  - Whitelist — match one or more ID values (count and width upper bounds are design parameters).

  - Blacklist — exclude listed IDs.

  - All IDs (same effect as ID filter off).

MAY

- Number of whitelist/blacklist entries; mask/range vs exact match lists.

### 4.3 Direction Filter

MUST

- Register-selectable:

  - Read only

  - Write only

  - Read and Write

MAY

- Whether “Write” covers AW only vs AW+W channels per AXI rules.

### 4.4 Applicability Summary (MUST)

| Feature | Filter combination |
| --- | --- |
| Bandwidth logging | Address + ID + DIRECTION |
| Latency logging (§5) | Same |
| Bandwidth throttling (§6) | Address + ID + DIRECTION (same combination as latency logging) |

## 5. Performance Logging

### 5.1 Bandwidth Logging

Summary (MUST)

- Period: Select cycle count or transaction count (AW+AR handshake definition in register documentation). Repeats while enabled.

- Measurement: Derive bytes from AxSIZE × burst length, etc.; accumulate Write and Read separately; push to FIFO on period end.

- FIFO: Depth is a parameter; Even/Odd banks; switch storage bank every N periods (N < depth, set by control register).

- Interrupt: Optional level interrupt every N periods; mask via interrupt enable.

- Status: Current Even/Odd write target; which FIFO bank was written before the last interrupt.

- Filters: Count eligibility per §4.

MAY

- FIFO implementation (memory type), metadata fields.

### 5.2 Latency Logging

#### 5.2.1 Period and Average Storage

MUST

- Use the same period registers as bandwidth logging.

- Single-transaction latency: store per-period averages only (sum / transaction count).

- Write latency: cycles from completed AW handshake until first B acceptance.

- Read latency:

  - From AR until first RVALID

  - From AR until last RLAST & RVALID (both statistics separately).

- Even/Odd FIFO and N-period switching: same policy as bandwidth logging.

MAY

- Internal structures (e.g. per-ID queues) as appropriate for the implementation.

#### 5.2.2 Read / Write Multi-Transaction Latency

Goal: When outstanding transactions overlap, store end-to-end delay of a “burst segment” and accumulated bytes consistent with the Period/FIFO policy.

MUST — Read multi-transaction latency

- Segment start: For a filtered Read, when no outstanding Read exists, start the latency counter when ARVALID & ARREADY complete.

- Segment end: For the last Read transaction of that segment, stop when RLAST & RVALID & RREADY complete.

- Storage: On completion (or per §5.2.3 at period boundaries), store byte count and period (cycle count) per the chosen storage mode (FIFO / average, etc.) using the same Even/Odd FIFO scheme as §5.1 and §5.2.1.

- If outstanding Reads already exist when a new Read starts: do not reset the multi-transaction latency counter; only add bytes (same segment).

MUST — Write multi-transaction latency

- Same policy as Read. Segment start: first valid request when no outstanding Write (typically AW or first valid per protocol). Segment end: BVALID & BREADY for the last Write of the segment.

- Additional Writes in the segment add bytes without resetting the counter.

MAY

- Byte accounting for Write multi segments (AW-only vs AW+W data) — must stay consistent with document-wide byte definitions.

- Whether multi-transaction stats go into period averages or per-sample FIFO — specify in register map.

#### 5.2.3 Multi-Transaction Statistics vs Period (MUST / partial MAY)

MUST

- Single- and multi-transaction latency stats include only transactions passing §4 filters.

- MAY share the same period timebase as bandwidth/latency logging.

MAY

- If multiple multi-transaction segments complete in one period, whether to store average, sum, or max — define in register map.

## 6. Bandwidth Throttling

### 6.1 Common

MUST

- Provide a throttle period register separate from logging period (period_throttle, naming MAY vary).

- Byte accounting for throttling uses the same byte rules as logging.

- Scope: Throttling applies only to transactions satisfying §4 address + ID + DIRECTION.

- Insert delay to limit bandwidth.

MAY

- Sliding vs fixed aggregation windows.

### 6.2 Threshold Mode Options (MUST)

Selectable dynamically by register.

#### 6.2.1 Static Threshold

MUST

- Each period_throttle window, if accumulated bytes exceed the configured threshold, insert delay on AW/AR (and §6.4) acceptance timing to reduce bandwidth.

- Different delay values may be set for AW vs AR.

#### 6.2.2 Credit-Based Threshold

MUST — Concepts

- credit_period: Independent of logging/static throttle period; register-programmable.

- Each credit_period, add number_of_credit (register) to the accumulated credit pool.

- Clamp accumulation to a maximum (register or parameter).

- On transaction request acceptance (VALID & READY on that channel — document per channel).

- Debit amount = request bytes right-shifted by credit_divider, i.e. (  / 2^{} ).

- Never let debit drive balance below zero (insufficient credit → §6.2.3).

- Separate credit pools for Read and Write.

MAY

- Credit counter width; ordering when multiple transactions debit in one cycle.

#### 6.2.3 Insufficient Credit — Delay (MUST)

MUST

- If credit is insufficient, delay acceptance to limit bandwidth.

- Apply independently to Read and Write paths (matching separate credits).

### 6.3 External Throttle Control

Receive throttle intent on pins separate from AXI, and apply insertion delay on the request path using delay registers independent of §6.2 and §6.4.

#### 6.3.1 Input Semantics (MUST)

MUST

- Port request_throttle[1:0] (§2.4).

- request_throttle == 0: No additional delay from this input (even if feature enabled).

- request_throttle == 1, 2, 3: Each selects a different delay on the request acceptance path. Delays are register-programmable and MUST use registers separate from AW/AR/W delay and credit settings in §6.2 and §6.4 — e.g. three dedicated registers (or equivalent) for encodings 1–3.

- Enable/disable of this feature MUST be dynamically programmable via APB. When disabled, request_throttle MAY be ignored (SHOULD document).

MAY

- Scope of “request signals”: typically AR/AW address-phase acceptance; extending to W/R response channels is MAY — document in register map.

- Three separate registers vs encoded table for delays 1–3.

#### 6.3.2 Interaction with Internal Bandwidth Throttling (MUST / MAY)

MUST

- When external throttle and §6.2 internal throttling are both active, combined delay MUST preserve AXI protocol and avoid deadlock.

MAY

- Rules for sum, max, priority — document in register map / implementation.

#### 6.3.3 Scope (MAY)

- Whether §4 filters also gate external throttle is MAY; optional register to select scope is MAY. Implementations that apply external throttle globally (all transactions) are allowed.

### 6.4 Write Path — WREADY Delay

MUST

- Support additional WREADY delay as part of throttling write data.

- Delay magnitude MUST be a separate register.

- Interaction with Static/Credit (e.g. WREADY delay only when credit-starved) MUST be documented in the register map; MUST comply with AXI W channel rules.

MAY

- Priority and combining rules between AW delay and W delay.

### 6.5 Mode Select and Registers (MUST)

MUST

- Select one of §6.2.1 or §6.2.2 at run time.

- Unused fields MAY be ignored when inactive (SHOULD document).

## 7. Address Protection

MAY be enabled independently of bandwidth throttle and latency log.

### 7.1 Protected Regions

MUST

- Define allowed regions as (base address, end address) pairs; maximum number of pairs is a design parameter.

- Each pair MAY be dynamically valid-gated.

- If a transaction address falls in no active pair, treat as protection violation (compare ARADDR/AWADDR; details MAY).

### 7.2 Graceful Ignore Behavior

MUST — Read

- Log AXID and ARADDR to error log registers.

- Complete handshakes without breaking AXI timing upstream or downstream.

- How illegal accesses are blocked from the downstream slave (e.g. no AR downstream + local error response) is MAY; no deadlock and VALID/READY compliance are MUST.

MUST — Write

- Log sufficient fields (AWADDR, AWID recommended).

- On the downstream write path, drive WSTRB = 0 for all bytes (MAY align/pack cycles; MUST keep protocol legal).

### 7.3 Error Log Register Format (MUST)

Each error source (or shared register) MUST include:

- valid: Error pending.

- multi: Encodes:

  - valid=1, multi=0: one occurrence.

  - valid=1, multi=N (N>0): N occurrences (saturate at multi width; MAY add overflow flag — document).

MUST

- Clear error logs via write-one-to-clear (W1C) through the IP’s APB CSR slave.

MAY

- Separate vs merged registers for Read/Write errors.

## 8. Interrupt and Status (summary)

- One interrupt line.

- MAY mask logging FIFO events, address-protection errors, etc., provided software can still distinguish events.

## 9. Verification (MUST)

- Sufficient assertions (SVA, etc.) for AXI protocol and counter boundary conditions in this document.

## 10. Structural Overview

### 10.1 Block Diagram

AXI Performance Monitor conceptual blocks

### 10.2 Filters → Features (reference Mermaid)

flowchart LR
  subgraph filt [Filter stage]
    A[Address]
    I[ID mode]
    D[Direction]
  end
  filt --> BW[Bandwidth log]
  filt --> LAT[Latency incl multi-tx]
  filt --> TH[Throttle static or credit]
  XTH[External request_throttle] --> TH
  filt --> AP[Address protection]

## 11. Implementation Deliverables (SHOULD)

The IP provider SHOULD supply, together with the implementation (directory layout MAY vary):

- RTL sources: Verilog/SystemVerilog suitable for synthesis and simulation.

- User documentation: Control register descriptions (bit meanings, addresses, reset values) consistent with this specification, plus clock/reset and parameter descriptions.

- Verification collateral: SVA assertions, test plan or verification summary.

## Appendix A. Terms, Abbreviations, and AXI Channels

This appendix summarizes concepts for readability; protocol details follow AMBA AXI.

| Term | Description |
| --- | --- |
| Upstream / Downstream | Relative to traffic flow through the monitor; connects to upstream masters/slaves on one side and downstream on the other (matches “Master Up / Master Down” naming in §2). |
| AR / AW / R / W / B | Read address, Write address, Read data, Write data, Write response channels. |
| Outstanding | A transaction whose response is not yet complete while subsequent address/data phases may be in flight; used in §5.2.2 multi-transaction segments. |
| APB | Low-speed configuration bus; this IP assumes a 32-bit APB slave for CSR access. |
| CSR | Control/status registers for configuration, statistics, and error logs. |
| Even/Odd FIFO | Two storage banks alternated every N periods so software can read one bank while the other continues recording. |
| W1C | Write 1 to Clear; writing 1 to a bit clears a flag. |
| SVA | SystemVerilog Assertions. |

Byte counting: Phrases such as “AxSIZE × burst length” refer to transaction byte volume derived from AXI AWSIZE/ARSIZE and AWLEN/ARLEN (and related rules); the exact formula SHOULD match the AXI standard and be stated in the register/user documentation.

Companion documentation: Register maps, pin lists, and timing are provided separately by the IP provider and MUST satisfy the functional requirements of this specification.